Suck it Up: Impervious Vs. Pervious Surfaces

A 5-Minute Watershed Activity for High School Students

By Elizabeth Rives

For Bridging the Watershed Educator Interview

May 13, 2010

Summary

This simple activity demonstrates the concept of impervious surfaces (primarily rooftops and roads) and their impact on watersheds.  Participants pour water on two downspout splash blocks – one covered with vegetation and the other with impervious material (to mimic roads) – and observe the contrasting water flow rates and volume.  Prior to the start of the activity, students are asked if they know what impervious means (not permitting penetration or passage; impenetrable) and if they can give explanations of how impervious surfaces would play a major role in urban watershed hydrology.

# Materials

2 downspout splash blocks

2 4x4 blocks ~ 6-8” in length (to raise back end of splash blocks)

2 1-liter Nalgene water bottles

Aluminum foil (for top of water bottle)

2 rubber bands (to hold aluminum foil onto mouths of water bottles)

2 small-lipped pans (to collect runoff)

2 watches (with second hands)

Vegetative matter in soil (slightly damp) to cover most surface area of one splash block

Material to represent impervious surface (e.g., plastic, aluminum foil)

Wide-mouthed mixing bowl with pouring spout

Waterproof tablecloth

# Objective

To demonstrate that precipitation that falls on rooftops and pavement quickly runs off, instead of infiltrating into the soil, as it would generally do in a natural or farmed landscape.  Increased impervious surfaces lead to increased runoff volume and peak flow rates, and subsequent increased magnitude and frequency of local flooding, soil erosion, and contaminant transport.

# Methodology

Students break into two groups, “P” (pervious) and “I (impervious).  Roles should be assigned to as many students as possible (timekeeper, water pourer, water measurer, observer, data keeper).  The two groups can perform the activity simultaneously or one group can follow another.

The pourer fills the Nalgene bottle with water to the 500 ml mark, places the aluminum foil top (pierced with 10 holes) around the mouth, and secures it by wrapping the rubber band tightly around the mouth.  Timer tells pourer when to start.  On the go signal, pourer pours entire 500 ml of water over the surface area of the splash block (explain they are attempting to simulate a rainfall event).  The timer records how long it takes for water to reach the drainage pan.  Group makes observations about water flow rate and volume, allowing the water to drain for 2 minutes (impervious group will not need any time to allow for drainage).

Each group separately pours the water from the pan into the large mixing bowl and pours it back into the Nalgene bottle.  Measurer measures water level and subtracts it from original 500 ml to determine runoff volume.

If time permits, students can add various pollutants (in small plastic bottles with olive oil; red wine vinegar (anti-freeze), sand (road sand, sediment), and road salt) to their surfaces and repeat the experiment, observing how much pollutant/sediment material flows into the pan.

# Goals/Discussion

- Students will define the term impervious and give examples of impervious surfaces.

- Students will theorize about the control factors in the experiment, such as amount of vegetation; rate/type of rainfall event; soil type, saturation, density, and compactness; and extrapolate to “real-world” situations.

- Students will observe flow rates and volume of water between impervious and pervious surfaces and describe ways to slow the rate and volume of water running into tributaries.

- Students will explain the effects of large volumes of fast-moving water and discuss the impact on waterways and aquatic wildlife.

- Students will discuss the economic and environmental impacts of the resulting damage to property and ecosystems.

- Students will discuss various stormwater management techniques, such as rain gardens, rain barrels, rooftop gardens, permeable paving, trees in parking lots, washing cars on grass, etc.